MERGE 4 DESIGN LLC 1212 Lobdell Avenue — Site Yield Analysis INTERNAL DRAFT — NOT FOR DISTRIBUTION ¦ May 2026

1. PROJECT INFORMATION

| Project Address | 1212 Lobdell Avenue, Wilmington, DE 19801 |
| --- | --- |
| Parcel Number | 2605130241 |
| Owner of Record | Wilmington Neighborhood Conservancy Land Bank Corp |
| Client | Wilmington Housing Authority (WHA) |
| Lead Firm | Merge 4 Design LLC (M4D) |
| Prepared By | David Ainsworth, M4D Predevelopment Lead |
| Date | May 2026 — Internal Draft |

2. PARCEL DATA

| Parameter | Value | Source |
| --- | --- | --- |
| Gross Frontage | 352.60 ft | NCC Parcel Record 2605130241 |
| Gross Depth | 175.00 ft | NCC Parcel Record 2605130241 |
| Gross Area | 61,705 SF / 1.417 AC | Calculated |
| Zoning District | 26R-3 | City of Wilmington Chapter 48 |
| Use Category | One Family Row Houses | 26R-3 permitted use |
| Flood Zone | Zone AE | FEMA FIRM Panel 10003C0156L, eff. 1/22/2020 |
| Base Flood Elevation (BFE) | 8.0 ft NAVD88 | FEMA FIRMette |
| Freeboard Requirement | 1.5 ft (18 in) | City of Wilmington floodplain ordinance |
| Minimum FFE | 9.5 ft NAVD88 | BFE 8.0 + freeboard 1.5 |

3. APPLICABLE ZONING STANDARDS — 26R-3 (Base R-3)

| Standard | Requirement | Notes |
| --- | --- | --- |
| Min. Lot Width — Interior Unit | 16 ft | Row house interior |
| Front Setback | 0 ft | Build-to line; consistent with row house context |
| Rear Yard | 20 ft | 3 stories or less |
| Side Yard — Interior Units | 0 ft | Party wall condition |
| Side Yard — End Units | 5 ft | Each end of row; applied to both ends |
| Max. Building Height | 35 ft / 3 stories | R-3 limit |
| Foundation Type | Slab on Grade | Zone AE — slab elevated to min FFE 9.5 ft NAVD88 |

4. SITE YIELD ANALYSIS

Net buildable frontage is calculated after deducting end unit side yards from gross frontage. Rear yard is deducted from gross depth to establish buildable depth. Two lot-width scenarios are evaluated per WHA direction.

| Parameter | Scenario A — 18 ft Lots | Scenario B — 20 ft Lots |
| --- | --- | --- |
| Gross Frontage | 352.60 ft | 352.60 ft |
| End Unit Side Yards (deducted) | 10 ft (2 × 5 ft) | 10 ft (2 × 5 ft) |
| Net Buildable Frontage | 342.60 ft | 342.60 ft |
| Lot Width | 18 ft | 20 ft |
| Unit Count | 19 units | 17 units |
| Gross Depth | 175 ft | 175 ft |
| Rear Yard (deducted) | 20 ft | 20 ft |
| Net Buildable Depth | 155 ft | 155 ft |
| Attached Garage Depth (est.) | 20 ft | 20 ft |
| Habitable Building Depth | ~135 ft | ~135 ft |
| Unit Footprint | 2,430 SF | 2,700 SF |
| Est. GFA / Unit (2-story) | ~4,860 SF | ~5,400 SF |

5. ROM COST SUMMARY

ROM unit cost range: $240,000 – $270,000 per unit. Range reflects M4Di G0 benchmark for fee-simple townhouse construction in the Wilmington corridor, inclusive of hard costs, soft costs, and predevelopment. Zone AE fill premium not yet incorporated — see flags below.

| Scenario | Units | ROM Low | ROM High | ROM Midpoint |
| --- | --- | --- | --- | --- |
| Scenario A — 18 ft Lots | 19 | $4,560,000 | $5,130,000 | $4,845,000 |
| Scenario B — 20 ft Lots | 17 | $4,080,000 | $4,590,000 | $4,335,000 |

ROM confidence level: G0 (order-of-magnitude). Confidence range ±30%. Suitable for feasibility and grant narrative purposes. Not for use in financing commitments or contractor solicitation without further development.

6. FLAGS & OPEN ITEMS

FLAG 1 — Zone AE Fill Premium: Existing grade elevation at the Lobdell parcel has not been confirmed. If grade is below min FFE 9.5 ft NAVD88, engineered fill is required. Estimated cost impact: $8,000 – $20,000 per unit depending on cut-to-grade differential. A survey elevation must be obtained before ROM is advanced to G1.

FLAG 2 — Parcel Address Discrepancy: NCC parcel record lists address as 1210 Lobdell St. Project reference is 1212 Lobdell Ave. Confirm correct address of record with WHA before any permit or grant submission.

FLAG 3 — Zoning Standards — Verification Required: 26R-3 dimensional standards applied from base R-3 (Ch. 48). Min lot width (16 ft), front setback (0 ft), and side yard (5 ft end unit) should be confirmed directly against current Chapter 48 text prior to permit application.

FLAG 4 — Parcel Configuration: This analysis assumes a single linear row of townhouses along the full Lobdell Ave frontage. WHA's 30-unit target suggests the parcel may support a more complex layout (double-loaded rows, rear alley access, or secondary row). Full site plan study required to evaluate configurations beyond this baseline.

FLAG 5 — NFIP Compliance: Zone AE designation requires NFIP-compliant construction. Flood insurance will be mandatory for any federally-backed buyer financing. Disclose to WHA for homebuyer program planning.

7. NEXT STEPS

- Obtain survey elevation at Lobdel parcel — confirm existing grade vs. BFE 9.5 ft NAVD88 min FFE

- Confirm parcel address of record with WHA (1210 vs. 1212; St. vs. Ave.)

- Verify 26R-3 dimensional standards directly against current Ch. 48 text

- Advance site plan study to evaluate double-loaded or multi-row configurations toward WHA 30-unit target

- Advance ROM to G1 once survey elevation confirmed and site layout resolved

- Incorporate Zone AE fill premium into G1 ROM once grade delta is known

INTERNAL DRAFT — M4D WORKING DOCUMENT — NOT FOR CLIENT DISTRIBUTION  |  Merge 4 Design LLC  |  215 N. Market Street, Suite 317, Wilmington, DE 19801